LWA-SV Memo 1

# LWA-SV F-engine firmware overview

D. Price 
28th January 2016

# Introduction

The LWA-SV digital signal processing system is known as the Advanced Data Processor (ADP).  It consists of:

- 32x CASPER ADC16x250-8 digitizer cards (a total of 32x16=512 inputs).

- 16x CASPER ROACH2 FPGA processing boards (Xilinx Virtex-6 SX475T FPGA)

- Mellanox SX1024 10/40GbE switch

- 6x GPU servers (ASUS ESC4000 G3 server)

The digitization, channelization, and channel selection are done on the ROACH2 boards, the firmware of which is written using the CASPER / MATLAB / Simulink / Xilinx ISE toolflow.

This memo gives an overview of LWA-SV firmware, by providing a walkthrough of the firmware’s Simulink diagram. It is intended that this document is read while clicking through the Simulink diagram, as it only provides overview of discrete blocks, and does not provide any insight into how the blocks fit together.

## Model walkthrough

The model consists of three top-level blocks:

- ADC – this contains the ADC16x250 digitizer yellow block, and reset / synchronization pulse logic.

- FFT – this contains the polyphase filterbank implementation for each of 32 inputs, and the post-channelization 4-bit requantization logic.

- PKT – this contains 10 GbE Ethernet packetization logic, including channel selection.

## ADC top-level block

### Reset logic

Main reset register adc_rst provides a reset line for ‘cores’ (i.e. logic blocks), and counter values. The adc_rst line should be set high (e.g. 0b11) and then low (0b00) to trigger a reset.

### Pulse-per-second and sync pulse

This logic is used to derive a ‘synchronization pulse’, that is required to reset internal logic of many CASPER blocks. Only one sync pulse will be generated and passed to the timing_sync goto block, and a counter reset is required to trigger a new pulse to be propagated on timing_sync.

adc_sync_in is a GPIO connection that is connected to a pulse-per-second signal derived from GPS. A global goto sync_pps is used in the packetizer to make sure values only change on a PPS, not mid-second.

The sync_out  GPIO propagates the sync pulse to the GPIO output (this is not used at LWA-SV). The sync_count register stores a count of how many PPS have occurred since the last counter reset.
The sync_pulse register stores how many timing sync pulses have been sent.

## FFT top-level block

### Sub-level PFB blocks

The PFB/FFT for the 32 input signals is split into 4 sub-blocks,  f0 – f3.  The output of each is a sync_rnd signal, which is the propagated sync pulse, and eq_out, which is all 8 requantized 4-bit input crammed together into a 64-bit word.

Inside each sub-block is 1x PFB FIR frontend, and 2x FFTs. Both of these are blackboxed.

The core of each fft sub-block is of course an FFT. This is blackboxed (so in a separate simulink diagram), but has 13 stages (2^13 point FFT), and is a biplex_real_4x CASPER block.

As the F-engine is a polyphase filterbank, the FFT is preceeded by an FIR low-pass prototype filter. This is also blackboxed.

Also under the FFT block is a 4-bit requantizer. This takes the 18_re_18_im signal from the FFT and converts it down to 4_re_4_im in the range [-7, 7]. In order to select the relevant bits, the signal is pre-multiplied on a per-channel basis, by a value written to a shared BRAM fft_f[0]_cg_bpass_bram.

The final item in under a FFT block is a shared BRAM fft_f0_rms_mon_4b, which is essentially a 4-bit, single integration spectrometer. It allows the 4-bit quantization value to quickly tuned by providing an instantaneous snapshot of quantized values.

## PKT top-level block

The packetizer selects channels to be sent out over 10GbE and

The packetizer has a whole bunch of user-configurable registers. The sync_change and change_on_sync blocks are used so that values only change at the start of a PPS/FFT/sync window. The user-configurable registers are:

| Packetizer control registers |  |
| --- | --- |
| pkt_roach_id | Roach ID number. Should run 1-16, used to identify ROACH |
| pkt_gbe0n_chan_per_sub | Number of channels per subband for gbe0 packetizer. Allowable values are 10-144 |
| pkt_gbe1_n_chan_per_sub | Number of channels per subband for gbe1 packetizer. Allowable values are 10-144 |
| pkt_gbe0_n_subband | Number of subbands for gbe0 packetizer. Total number of channels sent will be n_subband x n_chan_per_sub. Allowable values are 1-32 |
| pkt_gbe1_n_subband | Number of subbands for gbe1 packetizer. Total number of channels sent will be n_subband x n_chan_per_sub. Allowable values are 1-32 |
| pkt_gbe0_start_chan | Start channel (lowest channel in range) for gbe0. Allowable values 10-4000 |
| pkt_gbe0_start_chan | Start channel (lowest channel in range) for gbe1. Allowable values 10-4000 |
| pkt_gbe0_stop_chan | Stop channel (highest channel in range) for gbe0. Allowable values 20-4095 |
| pkt_gbe1_stop_chan | Stop channel (highest channel in range) for gbe1. Allowable values 20-4095 |
| pkt_tx_enable | Enable data flow. Controls data flow for both gbe0 and gbe1. LSB is for gbe0, LSB+1 is gbe1. To turn both on, write a value 0b11 = 3. To turn on gbe0 write 0b01=1 or for gbe1 0b10=2 |

| Packetizer control BRAMS |  |
| --- | --- |
| pkt_gbe0_ip_addr_bram pkt_gbe1_ip_addr_bram | List of IP address to send to. This should be the same length as the n_subbands; each subband is sent to a corresponding IP in this list. |
| pkt_gbe0_ip_port_bram pkt_gbe1_ip_port_bram | List of IP ports to send to. This should be the same length as the ip_addr_bram. |

| Packetizer output registers |  |
| --- | --- |
| pkt_gbe0_oflow_cnt pkt_gbe1_oflow_cnt | This register will be >0 if there’s overflows happening in the 10GbE core. This probably means that your n_subbands * n_chan_per_sub is too high. Only used in debugging. |
| pkt_gbe0_eof_cnt pkt_gbe1_oeof_cnt | Count of how many end of frame (EOF) have passed. More simply, the number of packets send out over the 10GbE link. Only used in debugging. |
| pkt_fifo_pc_full | A fractional number showing what percent of the FIFO is full. Only used in debugging. |
| pkt_gbe0_linkup pkt_gbe1_linkup | A register that shows if the 10GbE core is configured and the link is up. Returns 1 if up, 0 if down. |
| pkt_gbe0_full pkt_gbe1_full | Returns 1 if the gbe0 or gbe1 blocks are filled. |

https://casper.berkeley.edu/wiki/Main_Page